//Include Libraries

//#include <libraryname.h>

//Initialize LCD screen and RF driver

LiquidCrystal_I2C lcd(0x27,16,2);

RH_ASK driver;

//Define pin inputs

//#define pin_name pinnumber

#define SW1 1                     //pin 1 = joystick 1 button

//Define variables

//joystick setup

//int joystick_direction = pinnumber

int joystick1_x = A0;              //joystick1 x direction pin

//int variable_name;

int pos_x;                         //declaring variable to store horizontal value joystick 1

//int variable_name = initialized;

int high_x1 = 600;                 //variable high_x1 initialized at 600

//Create strings

//String str_stringname;

String str_command;

void setup() {

Serial.begin (9600);

//set pin modes to either input or output

//pinMode(pin_name, INPUT or OUTPUT);

pinMode(Red_led, OUTPUT);

pinMode(Blue_led, OUTPUT);

//Necessary code for RF 433 module

if (!driver.init())                     //Setup for 433 module

Serial.println("init failed");

//Necessary code for LCD screen

lcd.init();

lcd.backlight();

//Print Opening LCD screen message for 2000 ms and clear

lcd.setCursor(0,0);

lcd.print("Welcome to");

lcd.setCursor(0,1);

lcd.print("RoboSub V1.0");

delay(2000);

lcd.clear();

}

void loop() {

//Print lcd message to indicate system is ready (if desired)

//set up joystick input values

//direction = analogRead (joystick_direction)

pos_x = analogRead (joystick1_x) ;     //Read values from joystick 1

//-------Steering (left and right)-------

//----------checking horizontal values-------------

//----------LEFT--------------

// if statement to check which direction joystick is facing

//if (joystick_direction < specified number) then do...

if (pos_x < low_x1)

{

//Send left command to let receiver know what to expect

//create a string to tell which motors to engage

str_command = "11";                 // on the receiving side, "11" corresponds to the left motors

//Find amount of force in left direction

//map the amount to left joystick is pushed (400-0 joystick values = 0 - 150 pwm values)

int xleft = map(pos_x, 400, 0, 0, 150);

//Add 100 to previous value to keep digit count the same regardless of joystick value

xleft = xleft + 100;

//convert xleft to a string

str_amount = String(xleft);

//combine command and amount strings to send a single packet to receiver

str_out = str_command + "," + str_amount;

//Serial.print is useful for debugging. Use this to ensure the code is doing what you expect

Serial.print("LEFT");

Serial.println(str_amount);

//Create the message to be sent

static char *msg = str_out.c_str();

//Send message using driver

driver.send((uint8_t *)msg, strlen(msg));

driver.waitPacketSent();

//reset variable to zero

xleft = 0;

//short delay so receiver can get full message before receiving another

delay(100);

}

//-----------RIGHT------------

else if (pos_x > high_x1)

{

//Same as left directions with changes in command string and map function

//Send right command to let receiver know what to expect

str_command = "12";

//Find amount of force in right direction (600-1023 joystick values = 0-150 pwm values)

int xright = map(pos_x, 600, 1023, 0, 150);

xright = xright + 100;

//convert xright to a string

str_amount = String(xright);

//combine command and amount

str_out = str_command + "," + str_amount;

Serial.print("RIGHT");

Serial.println(str_amount);

static char *msg = str_out.c_str();

driver.send((uint8_t *)msg, strlen(msg));

driver.waitPacketSent();

xright = 0;

delay(100);

}

//---------STRAIT----------

else

{

//Same as left and right but with differnce in command string

}

// ----------Motion (forward and backward) -------

//------FORWARD----------

//--------checking vertical values------------

if (pos_y < low_y1)

{

//Same as left but with difference in command string

}

//-------BACKWARD--------

else if (pos_y > high_y1)

{

//Same as right but with difference in command string

}

//------------STOP----------

else

{

//Same as strait but with difference in command string

}

//----------Buoyancy (Up and Down)------------------

//----------UP-----------------

//--------checking vertical values------------

if (pos2_y < low_y1)

{

//Same as left but with difference in command string

}

//-----------DOWN----------------

else if (pos2_y > high_y1)

{

//Same as right but with difference in command string

}

//----------STOP UP-------------

else

{

//Same as strait but with difference in command string

}

// --------------Center Button Pushed -----------

//Can use while loop to do specified actions using push button, must be careful not to get stuck in it

while (state == LOW)

{

digitalWrite(Red_led, HIGH);

break;

}

while (state == HIGH)

{

digitalWrite(Red_led, LOW);

break;

}

}

//----------------Functions------------------------

//Create functions

//void function_name(){}

void Eascend()

{

//Do stuff here

}